- SPECIFICATION

- FD BLOUSE COAT, DOUBLE BREASTED

- STYLE #:	#546MNV – male, navy blue

STYLE:  Double breasted, 6 button front, peak lapel, closed back (no vent), plain barrel cuff (no buttons or vent), fully lined, swing action back inside lining with stretch mesh, two (2) inside reinforced breast pockets, badge tab, optional lower pocket flaps and epaulets included with each garment that can be sewn on.

- FABRIC:

- Contents: 100% polyester (to match #600MNV trouser)

- Construction:  Warp = 64, Filling = 54

Yarns:	Texturized

Weave: 2x1 twill

Weight: 6.9 ozs./sq. yd., 11.50 ozs./linear yd.

Dying: High-pressure jet dyed

Finish: Soil release

- POCKETS:

There shall be two (2) inside breast pockets, one on each front panel.  These shall be Reece double welt style, 5-1/4” wide.  The welt shall be made of shell material. They will be fully faced with shell material, 1” all around the welt opening. The pocket bag shall be made of nylon taffeta and shall measure 5 ½” deep.

LINING:  The blazer shall be fully lined with nylon taffeta.  Seams of this lining shall be serge seamed on the inside to prevent raveling.

SWING ACTION PANEL:  There shall be an expandable knit mesh panel set between the back lining and the sleeve, from the shoulder seam to the underarm.  This shall measure approximately 1 ¾” wide.  The purpose will be to allow the lining to fully expand when the arms are moved forward.

FRONT:  There shall be a peak lapel.  The bottom (wide) peak shall measure approximately 3-3/4”” wide; the top (narrow) peak shall measure approximately 2-3/4” wide.  The bottom of the coat will be straight with square fronts (not rounded).  Note that each front is actually two pieces (there is a side panel).

COLLAR & LAPEL:  The collar shall be underlined with shell material and top stitched 3/16”.  The lapel shall be a peak style and top stitched 3/16”.  The top peak shall be 2-3/4” wide.  The bottom peak shall be 3-3/4” wide.

SLEEVES:  The sleeves shall be two-piece, set in style.  They shall have an industry standard sleeve head and shoulder pad.  There will be a barrel cuff with no sleeve vents or buttons.  *** There shall be at least 3” extra shell & 3” extra lining fabric inside the sleeve hem allowing for alterations (sleeve lengthening).

BACK:  The back will be a two-piece back with center seam and 3/16” topstitching.

SHOULDER PADS: There shall be good commercial grade shoulder pads with no visible puckering.

BUTTONS:  There shall be six (6) 36 ligne silver metal FD buttons on the front of the coat.  These buttons shall be solid brass, nickel coated.  A steel back will not be accepted.  The face of the button shall be a dome shape.  The back shall be a wire eye (loop) and they shall be securely sewn on.  There shall be one (1) 36 ligne, 4-hole plastic button on the inside left front to secure to a corresponding buttonhole on the right front.

HEM OPENING: There shall be an opening on both the left and right fronts, just beyond the facing material.   Through this opening, you can insert your hand inside between the lining and the shell material to remove the metal buttons.

SEAMING:  Except as noted above, all seams shall be single needle, pressed open.

THREAD:  All thread shall be poly-core, color fast, 2-ply.  This shall identically match the color of the blazer.

INTERLINING:  The fronts shall have commercial quality fusing.  All commercially accepted shoulder pads, sleeve heads, bridal tapes, fusing and interlining shall be used throughout.  The sleeve hems shall be interlined with good quality canvas for firm body.

FINISHING:  This garment shall be finished in a first class workmanlike manner with all loose threads removed.  The omission or apparent omission of any wording or specifications shall mean that only the highest quality industry standards shall be used on all fabric and component parts including labor.  Where there may be a discrepancy between the standard sample and the written specification, the detail of the standard sample shall prevail.

LABELS:  All labels will be permanent to withstand repeated cleanings for the life of the garment.  The following labels shall be applied:

MAIN LABEL:  There shall be a main label.  This shall identify the manufacturer either by name & location, registered trademark or WPL or RN number as required by the Federal Trade Commission.

CONTENTS/CARE/SIZE LABEL:  There shall be an additional label on both the main jacket and removable liner clearly showing the fabric contents, laundering instructions and garment size.

PACKING & FOLDING:  Blazers will be professionally pressed and shipped on hangers, flat packed. They shall be in polyester bags.

SIZES:

Regular: 34-60

Longs: 38-60