#575MFL ANSI-3, THREE SEASON JACKET

\

Inside SOFT SHELL jacket/liner               Main label for both shell and liner

STYLE:

- Average 29” (size LG).

- Zipper front to top of collar

- Outside storm fly with Velcro closure (4-5 spots), over zipper

- Zip-out liner is also a self-closing jacket.

- Hidden, unlined, pop-out hood.

- Segmented silver reflective tape per diagram.

MATERIAL:

OUTER JACKET:

SHELL:

- ANSI 3 compliant

- 100% nylon oxford with waterproof back coating

- Seam sealed – waterproof and windproof

- Upper front = fluorescent yellow

- Lower front = black

- Lower sleeves = black all around

- Back = fluorescent yellow

LINING:

- 100% nylon

INNER ZIP-OUT LINER JACKET:

SHELL:

- Soft shell, 100% polyester, 4-way stretch, waterproof/breathable membrane, windproof

- Black

DESIGN DETAILS:

OUTER JACKET:

- Zipper front to top of collar, Vislon, #5, black

- Outside storm fly with Velcro closure.

- Two lower 2-way pockets with center inverted pleat.  Straight bottom flap, mitered corners, Velcro closure under each side.  Side opening hand warmer pockets lined with fleece material on top side.

- Two upper vertical, welt storage pockets.  Opening to be 6-1/2” wide.  Welt cover to be ¾”.  Hidden zipper to be nylon coil #5, black.  Bar-tack top & bottom of welts.

- One left chest welt pen pocket, 1-1/2” wide and 5” deep.

- One inside horizontal set-in pocket

- Zipper side vents (11” tall), #5 nylon coil zipper, black.  There will be a tab & snap closure at the bottom of the side vents.

- Elasticized cuffs with Velcro tab closure.

- Microphone loops at shoulder on each side.

- Hood with visor & draw cord.  There will be a zipper under the collar and the hood will drop down inside the jacket between the shell and the lining.

- Waistband to be shirred elastic around back.

INNER LINER JACKET:

- Inside liner becomes a self-wearing jacket.  Zippers shall coordinate with a zipper inside each side of the shell jacket.  There shall be a loop at the back of the neck.  This shall secure to a tab & snap inside the neck of the shell jacket.

- Zipper front to top of collar, coil, #5, black

- Two lower side seam slash welt pockets.

- One upper vertical left chest pocket

- Bottom shall be straight hem, no knit waistband.  Back will be shirred elastic.

- Zippered side vents (11”) to coordinate with zipper side vents on main jacket.

- There will be a tab and snap closure at the bottom of the side vent zippers.

- 2-1/2” wide cuff made of shell fleece material.

REFLECTIVE TAPE:

There shall be segmented silver reflective tape on the yellow shell jacket as follows:

- Around upper chest

- Around lower body

- Around upper sleeves

- Around lower sleeves

- Over each shoulder from/to the upper chest

SIZES:

XS to 4X and 6X