SPECIFICATIONS – Top Brass® trousers

STYLE #:	609MBK – male, black

609MNV – male, navy blue

609FNV – female, navy blue

STYLE:  Uniform trouser, four pockets, zipper fly front, seven beltloops, 2” waistband, Snugtex, Ban Rol, crotch lining, French fly, triangular bartacks, set-in belt loops

- FABRIC:

- Contents: 100% Dacron® polyester

- Construction:  Warp = 67, Filling = 57

Weave: 2x1 right hand gabardine

Weight: 12-12 ¼ ozs./lin.yd.

6.85 ozs./sq. yd.

Yarns: Texturized

Dying: High-pressure jet dyed

Finish: Soil release

Tensile strength: 215 x 191 lbs.

Slippage:	44 x 50 lbs.

Tear strength: 15.5 x 13.5 lbs.

Pilling (5 wash): 4.0

Appearance:	3.5

Hot Head Shrinkage: 2.4 x 1.4 %

Shrinkage (5 wash): 1.4 x 0.9 %

# COLORS #:	609MNV – male

609FNV - female

POCKETS: There shall be two (2) front quarter top styled pockets.  The swing type pocket bag shall be 7 ½” deep.  The pocket shall angle forward 1 ½ to 1 ¾” at the top.  The pocket facing attached to the side seam shall be 2 ¾” wide at the top.

There shall be two (2) rear hip pockets.  The left shall have a button and tab closure.  They shall be 5 ¼” wide.  The left pocket shall have a button and hole closure.  This shall be a Reece, double-welted style pocket.  The pocket bag shall be 6” deep.  These welts shall be top-stitched top and bottom.

POCKETING:  Shall be 50% polyester, 50% cotton, 260-weight drill and shall withstand all pilling for a minimum of 30 wash cycles.  Color shall be black.

WAISTBAND:  Male waistband shall be approximately 2” wide with Snugtex® and Ban Rol®.  There shall be a curtain made of the same fabric and color as that of the pocketing.  On the outside of the waistband, along the seam where the waistband is joined to the body of the trouser, there shall be a topstitch to secure inside the curtain. Female waistband shall be the same as above, with the following exceptions:  Replace Snugtex® with elastic shirring from side seam to side seam.

BELTLOOPS:  There shall be seven (7) beltloops (except 5 on sizes 28-31 male and 6 female).  These shall be of shell material and interlined.  They shall be set into the top and bottom of the waistband seam.  They shall be ¾” wide and 2” long.  The belt loop at the side seams shall be set to the back of the side seam 5/8” to facilitate striping if needed.

FRENCH FLY: There shall be a French fly, made of shell material.

CROTCH LINING: There shall be 2/4 front crotch lining, made of pocketing material.  These shall have folded edges, and made of double ply material.

ZIPPER: Shall be brass and of good commercial quality.  Shall withstand all laundering and pressing for the life of the garment.  Shall be self-locking style.

INSEAM:  Shall be proportionally graded to average 37” open bottom.

SEAMS: Special attention shall be paid to the seam construction:

Leg seams:  Shall be serged on each half and pressed open.

Seat seam: Shall be tandem needle sewn with a minimum 2 ½” outlet.

BARTACKS: There shall be straight bartacks at the top and bottom of the quarter top pockets, the base of the fly and one bartack securing the inside of the right fly to the left fly about 1” above the bottom of the zipper. There shall be triangular bartacks at the ends of the hip pockets.

THREAD:  All thread shall be color fast, polyester core, two ply.  This shall identically match the fabric color.

FINISHING:  This garment shall be finished in a first class workmanlike manner with all loose threads removed.  The omission or apparent omission of any wording or specifications shall mean that only the highest quality industry standards shall be used on all fabric and component parts including labor.  Where there may be a discrepancy between the standard sample and the written specification, the detail of the standard sample shall prevail.

LABELS:  All labels will be permanent to withstand repeated cleanings for the life of the garment.  The following labels shall be applied:

MAIN LABEL:  This shall identify the manufacturer either by name & location, registered trademark or WPL or RN number as required by the Federal Trade Commission.

CONTENTS/CARE/SIZE LABEL:  There shall be an additional label clearly showing the fabric contents, laundering instructions and garment size.

SIZES:

Male:	28-38 (all sizes)

40-54 (even sizes)

Above 54 as needed

Female: 4-28