- SPECIFICTION

- DRESS SHIRT

- STYLE #:	          Long Sleeve			         Short Sleeve

- #780MWH, male, white		#781MWH, male, white

- #780FWH, female, white		#781FWH, female, white

STYLE:  Dress shirt, seven (7) button placket front, banded collar, one (1) plain patch pocket on the left front, barrel cuff with placket vent.

- FABRIC:

- Contents: 65% polyester, 35% cotton

- Construction:  Warp = 136, Filling = 72

Yarns: Warp = 30’s,  Filling = 30’s

Weave: Broadcloth

Weight: 3.1 ozs./sq. yd.

FRONT:  There shall be a full separate placket on the left front, 1 ¼” wide, sewn on, having one buttonhole on the collar stand and buttonholes on the front spaced an average of 3 3/4” apart.  The right front shall be plain with a minimum turned back facing of not less than 3/4”.  There shall be no raw edge on this facing.  19 ligne buttons shall be attached to the right front with corresponding buttonholes on the placket.

- POCKETS:

There shall be one (1) plain patch pocket.  It shall have a straight bottom with rounded corners. Topstitching shall be 1/32” on all sides of the pockets.  The patch pocket shall be 4 5/8” wide and 5 ¼” high.  There shall be a triangular tack in each corner of the pocket.

COLLAR:  There shall be a banded collar.  The points shall be 2 7/8” long.  There shall be collar stays inside the collar points.  The collar shall be top fused.  The interlining shall be woven polyester or canvas.  The collar shall be topstitched 3/16” from the edge and shall close with a 19 ligne button.

CUFF:  There shall be a barrel cuff on the long sleeve shirt, which shall be 2 7/8” wide with a two (2) button adjustable closure.  There shall be a placket vent with a center button.

YOKE & BACK:  There shall be a two (2) piece yoke, stitched and turned.  The yoke shall be 2” below the collar seam in the back.  The yoke shall be top-stitched front and back.

BACK CENTER BOX PLEAT:  There shall be a 1 ½” back center box pleat.

BUTTONS:  The buttons shall be first grade, pearlescent plastic, 4-hole, 19 ligne, to match the color of the shirt.  They shall be “X” stitched.

SEAMING:  Sleeves and side seams shall be double needle felled.  The armhole shall be single needle topstitched.  All remaining seams shall be single needle stitched not less than 10-12 stitches per inch.  Bottom hem of shirttail shall be continuous machine hemmed.

THREAD:  All thread shall be poly-core, color fast, 2-ply.  This shall identically match the color of the shirt.

INTERLINING:  The collar, collar band, placket front, cuffs, pocket flaps and epaulets shall be interlined with 100% woven polyester taffeta or canvas.

FINISHING:  This garment shall be finished in a first class workmanlike manner with all loose threads removed.  The omission or apparent omission of any wording or specifications shall mean that only the highest quality industry standards shall be used on all fabric and component parts including labor.  Where there may be a discrepancy between the standard sample and the written specification, the detail of the standard sample shall prevail.

LABELS:  All labels will be permanent to withstand repeated cleanings for the life of the garment.  The following labels shall be applied:

MAIN LABEL:  There shall be a main label.  This shall identify the manufacturer either by name & location, registered trademark or WPL or RN number as required by the Federal Trade Commission.

CONTENTS/CARE/SIZE LABEL:  There shall be an additional label on both the main jacket and removable liner clearly showing the fabric contents, laundering instructions and garment size.

PACKING & FOLDING:  Shirts will be professionally folded and packaged with a stand up collar.

SIZES – Male:

|  |  |  | Back |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Size | Neck | Sleeve | Length | Chest | # Waist | Sweep | Yoke |
| Small | 14-14 ½ | 32/33 | 33 | 43 | 38 | 41 | 17 |
| Medium | 15-15 ½ | 32/33 | 33 1/4 | 47 | 42 | 45 | 18 ½ |
| Large | 16-16 ½ | 32/33 | 33 1/2 | 51 | 49 | 51 | 20 |
| X-Large | 17-17 ½ | 32/33 | 33 3/4 | 55 | 53 | 55 | 21 |
| 2X-Large | 18-18 ½ | 32/33 | 34 | 60 | 58 ½ | 60 | 21 ½ |
| 3X-Large | 19-19 ½ | 32/33 | 34 1/4 | 64 | 62 ½ | 64 | 22 ¼ |
| 4X-Large | 20-20 ½ | 32/33 | 34 1/2 | 68 | 66 1/2 | 68 | 23 3/8 |
| 6X-Large | 22-22 ½ | 32/33 | 35 | 74 | 72 1/2 | 74 | 24 7/8 |

Sizes – Female:

|  |  |  | Back |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Size | Neck | Sleeve | Length | Chest | # Waist | Sweep | Yoke |
| 4 | 13 ½ | 23 ¼ | 28 ¼ | 39 ½ | 38 ¾ | 41 | 16 |
| 6 | 14 | 23 3/8 | 28 ½ | 40 ½ | 39 ¾ | 42 | 16 ¼ |
| 8 | 14 ½ | 23 ½ | 28 ¾ | 41 ½ | 40 ¾ | 43 | 16 ½ |
| 10 | 15 | 23 5/8 | 29 | 42 ½ | 41 ¾ | 44 | 16 ¾ |
| 12 | 15 ½ | 23 ¾ | 29 ¼ | 44 | 43 ¾ | 45 ½ | 17 ¼ |
| 14 | 16 | 23 7/8 | 29 ½ | 45 ½ | 44 ¾ | 47 | 17 ¾ |
| 16 | 16 ½ | 24 | 29 ¾ | 47 | 46 ¼ | 48 ½ | 18 ¼ |
| 18 | 17 | 24 1/8 | 30 | 49 | 48 ¼ | 50 ½ | 18 ¾ |
| 20 | 17 ½ | 24 ¼ | 30 ¼ | 51 | 50 ¼ | 52 ½ | 19 ¼ |
| 22 | 18 | 24 3/8 | 30 ½ | 53 | 52 ¼ | 54 ½ | 19 ¾ |
| 24 | 18 ½ | 24 ½ | 30 ¾ | 55 | 54 ¼ | 56 ½ | 20 ¼ |
| 26 | 19 | 24 5/8 | 31 | 57 | 56 ¼ | 58 ½ | 30 ¾ |
| 28 | 19 ½ | 24 ¾ | 31 ¼ | 59 | 58 ¼ | 60 ½ | 21 ¾ |

Back length will be proportionally longer for male sleeves 34/35 & 36/37.